HW Subsystem Test Plan

Sensor board is not connected to the Microcontroller-board, References: HW_Testing.docx (PM3), Checklist_Marks.xlsx (PM4)

| Test item, Description | Test Conditions | Expected, Tolerance | Measured Result | Comparison, Verdict | Remarks, Discussion |
| --- | --- | --- | --- | --- | --- |
| Signal Gain-Phase - w/o radar sensor - on I- and Q-paths | Sine input signal: 50 uVpp & 1mVpp 10 Hz … | Gain in passband: … dB ± … dB cut-off frequencies LP/HP: … | Scope Option: Bode Plots (see example) | passed / as designed not passed | … |
| DC level - w/o radar sensor - on I- and Q-paths - after each Amp |  |  |  |  |  |
| Radarsignal Gain-Phase - on I- and Q-paths | Radar Doppler Target K-DT1 with fd = … at distance … | I/Q-Gain-Difference … I/Q-Phase difference 90° ± … | Scope Channel A/B |  |  |
| Additional Features |  |  |  |  |  |

Example:  Frequency Response Bode Plot (PicoScope 2206B: https://www.picotech.com/downloads, 
3rd party WINDOWS app: FRA For PicoScope: https://bitbucket.org/hexamer/fra4picoscope/wiki/Home)

FW Subsystem Test Plan

Microcontroller-board is not connected to the Sensor board, References: HW_Testing.docx (PM3), Checklist_Marks.xlsx (PM4)

| Test item, Description | Test Conditions | Expected, Tolerance | Measured Result | Comparison, Verdict | Remarks, Discussion |
| --- | --- | --- | --- | --- | --- |
| ADC Samples - on I- and Q-paths | no signal … | DC-value of virtual GND … |  | passed / as designed not passed | … |
| FFT output - on display | Sine input signal: … Hz, … Vpp | peak and leakage |  |  |  |
| Frequency / Speed - w/o sign! - on I- and/or Q-paths | Sine input signal: … Hz, … Vpp | Frequency +… ± Δf/2 (Δf = fs/NFFT) | speed or frequency without sign |  |  |
| Frequency / Speed - with sign | sin/cos signals from Matlab and USB-Audio Interface: … Hz, … Vpp | Frequency … ± Δf/2 (Δf = fs/NFFT) | speed or frequency with sign |  |  |
| Additional Features |  |  |  |  |  |